Experiment 3 : Design of Clock Divider Circuit using RTL

Objectives:

To create and validate a clock divider by two circuit with RTL (Verilog HDL) that generates a 50 MHz output clock from a 100 MHz input base clock.

To create and validate a clock divider by four circuit with RTL (Verilog HDL) that generates a 25 MHz output clock from a 100 MHz input base clock.

To realize flip-flop toggle logic for divide-by-2 operation and a 2-bit synchronous counter for divide-by-4 operation with active-high asynchronous reset.

To carry out behavioral simulation in Vivado Simulator (XSIM), measure output clock periods, and verify frequency division factors.

Software Required:

Xilinx Vivado Design Suite, Vivado Simulator (XSIM), PC or laptop.

Procedure:

## (a) Generation of 50 MHz Clock from 100 MHz Clock (Clock Divider by 2)

1. Start a new RTL project in Xilinx Vivado targeting the appropriate FPGA device.

2. Add a Verilog module clock_divider_by2 defining ports: input wire clk, input wire rst, and output reg clk_out.

3. Write the RTL code with toggle logic (clk_out <= ~clk_out) triggered on the rising edge of clk or rst.

4. Create a testbench tb_clock_divider_by2 to generate a 100 MHz periodic clock signal (always #5 clk = ~clk;) and apply initial reset.

5. Execute behavioral simulation with Vivado Simulator (XSIM) and inspect the 50 MHz output clock waveform.

## (b) Generation of 25 MHz Clock from 100 MHz Clock (Clock Divider by 4)

1. Start a new RTL project or add a new Verilog module clock_divider_by4.

2. Define ports: input wire clk, input wire rst, output reg clk_out, and internal 2-bit register reg [1:0] count.

3. Write the RTL code incrementing count on posedge clk and toggling clk_out when count reaches 2'b01.

4. Create a testbench tb_clock_divider_by4 to generate a 100 MHz clock signal and apply initial reset.

5. Execute behavioral simulation with Vivado Simulator (XSIM) and inspect the 25 MHz output clock waveform.

Program

- Clock Divider by Two Module

module clock_divider_by2 (
    input  wire clk,
    input  wire rst,
    output reg  clk_out
);
    always @(posedge clk or posedgerst) begin
        if (rst)
clk_out<= 1'b0;
        else
clk_out<= ~clk_out;
    end
endmodule

- Testbench Program for Clock Divider by Two

timescale 1ns / 1ps
module tb_clock_divider_by2;
    reg clk;
    reg rst;
    wire clk_out;

    clock_divider_by2 uut (
        .clk(clk),
        .rst(rst),
        .clk_out(clk_out)
    );

    always #5 clk = ~clk; // 100 MHz clock generation

    initial begin
        clk = 0;
rst = 1;
        #20 rst = 0;
        #100;
        $stop;
    end
endmodule

- Clock Divider by Four Module

module clock_divider_by4 (

input  wire clk,

input  wire rst,

output reg  clk_out

);

reg [1:0] count;

always @(posedge clk or posedgerst) begin

if (rst) begin

count   <= 2'b00;

clk_out<= 1'b0;

end else begin

count <= count + 1'b1;

if (count[0] == 1'b1)

clk_out<= ~clk_out;

end

end

endmodule

- Testbench Program for Clock Divider by Four

`timescale 1ns / 1ps
module tb_clock_divider_by4;
    reg clk;
    reg rst;
    wire clk_out;

    clock_divider_by4 uut (
        .clk(clk),
        .rst(rst),
        .clk_out(clk_out)
    );

    always #5 clk = ~clk; // 100 MHz clock generation

    initial begin
        clk = 0;
rst = 1;
        #20 rst = 0;
        #1000;
        $stop;
    end
endmodule

Output

| S.No | Output Parameter | Theoretical / Expected Value | Simulated / Measured Value | % Deviation |
| --- | --- | --- | --- | --- |
| 1 | Input Clock Period (clk) | 10.0 ns | 10.0 ns | 0% |
| 2 | Divide-by-2 Frequency (clk_out1) | 50.0 MHz | 50.0 MHz | 0% |
| 3 | Divide-by-2 Clock Period | 20.0 ns | 20.0 ns | 0% |
| 4 | Divide-by-4 Frequency (clk_out2) | 25.0 MHz | 25.0 MHz | 0% |
| 5 | Divide-by-4 Clock Period | 40.0 ns | 40.0 ns | 0% |
| 6 | Output Duty Cycle | 50% | 50% | 0% |

Calculation

1. Clock Period Relationship: T = 1 / f

2. Frequency Division Formula: f_out = f_in / N

• Divide-by-2 Circuit: f_out1 = 100 MHz / 2 = 50 MHz  ==>  T_out1 = 20 ns

• Divide-by-4 Circuit: f_out2 = 100 MHz / 4 = 25 MHz  ==>  T_out2 = 40 ns

3. Percentage Frequency Deviation: Deviation (%) = | (f_simulated - f_theoretical) / f_theoretical | * 100

Figure 1: Clock division by 2

Figure 2: Clock division by 4

Result

The clock divider by two (50 MHz) and clock divider by four (25 MHz) circuits were designed and simulated successfully with RTL Verilog in Xilinx Vivado. Simulation results in Vivado Simulator (XSIM) confirm that the output clock frequencies are correctly divided from the 100 MHz input base clock as per the specified division factors (N=2 and N=4) with exact periods of 20 ns and 40 ns, validating 100% functional accuracy.