Experiment 6: Design of image Intensity Inversion circuit in Zynq SoC

Objectives

- To create a Verilog-based image intensity inversion module for functional simulation with Xilinx Vivado Simulator (XSIM).

- To process a 256 × 256 grayscale image containing 65,536 pixels with a Verilog testbench.

- To realize 8-bit grayscale intensity inversion with the equation (P_{out}=255-P_{in}).

- To validate the inverted pixel values and analyze the input and output signals with Vivado simulation waveforms.

Software Required

- Software: Xilinx Vivado Design Suite, Vivado Simulator (XSIM), and a text editor for preparing Verilog RTL and testbench files.

Procedure

A. Grayscale Image Pre-processing and Data Formatting

- Prepare a 256 × 256 grayscale image containing 65,536 pixels, with each pixel represented by an 8-bit unsigned integer ranging from 0 to 255.

- Convert the image into a sequential list of pixel values and save the values in image_in.txt, with one pixel value per line.

- Create a Vivado RTL project for behavioral simulation of the image intensity inversion module. FPGA board targeting and hardware implementation are not required.

B. RTL Modeling and Intensity Inversion Logic

- Develop the intensity_inversion Verilog module with an 8-bit pixel input and an 8-bit pixel output.

- Implement the intensity inversion equation:

pixel_out = 8'd255 - pixel_in

- If synchronous operation is required, update the output on the rising edge of the clock when valid_in is high. Include reset and valid_outsignaling if specified in the design.

- Verify the inversion operation with representative pixel values, such as:

  - Input 0 → Output 255

  - Input 100 → Output 155

  - Input 128 → Output 127

  - Input 255 → Output 0

C. Behavioral Simulation and Waveform Verification

- Develop a Verilog testbench (tb_intensity_inversion) to read pixel values from image_in.txt with $fopen and $fscanf.

- Apply the pixel values sequentially to the design and allow sufficient simulation time for the output to be generated.

- Record the corresponding inverted pixel values in image_out.txt with $fdisplay.

- Execute behavioral simulation in Vivado Simulator (XSIM) and inspect the input pixels, clock, reset, valid signals (if applicable), and output pixels in the waveform window.

- Confirm that each output pixel satisfies the equation pixel_out = 255 - pixel_in.

- Confirm that all 65,536 pixels are processed correctly and document the simulation results with representative input-output samples and waveform observations.

Program

- Verilog RTL Module

module intensity_inversion (
    input  wire        clk,
    input  wire        rst,
    input  wire [7:0]  pixel_in,     // 0 to 255
    input  wire        valid_in,
    output reg  [7:0]  pixel_out,
    output reg         valid_out
);
    always @(posedge clk or posedgerst) begin
        if (rst) begin
pixel_out<= 8'd0;
valid_out<= 1'b0;
        end
        else begin
            if (valid_in) begin
pixel_out<= 8'd255 - pixel_in;   // Intensity inversion
valid_out<= 1'b1;
            end
            else begin
valid_out<= 1'b0;
            end
end
end
endmodule

- Verilog Testbench Code

`timescale 1ns/1ps
module tb_intensity_inversion;
    reg        clk, rst;
    reg  [7:0] pixel_in;
    reg        valid_in;
    wire [7:0] pixel_out;
    wire       valid_out;

    integer infile, outfile;
    integer r;

intensity_inversion UUT (
        .clk(clk),
        .rst(rst),
        .pixel_in(pixel_in),
        .valid_in(valid_in),
        .pixel_out(pixel_out),
        .valid_out(valid_out)
    );

    // Clock Generation (100 MHz / 10 ns period)
    always #5 clk = ~clk;

    initial begin
        clk = 0;
rst = 1;
valid_in = 0;
pixel_in = 0;
        #20 rst = 0;

infile  = $fopen("image_in.txt", "r");
outfile = $fopen("image_out.txt", "w");
        if (infile == 0 || outfile == 0) begin
            $display("File open error");
            $finish;
        end

        while (!$feof(infile)) begin
            r = $fscanf(infile, "%d\n", pixel_in);
valid_in = 1'b1;
            @(posedge clk);   // input accepted
            @(posedge clk);   // output available
            if (valid_out)
                $fdisplay(outfile, "%d", pixel_out);
        end

valid_in = 0;
        #20;
        $fclose(infile);
        $fclose(outfile);
        $display("Intensity inversion simulation finished.");
        $finish;
    end
endmodule

Formulas

1. Image Intensity Inversion Relationship: P_out = 255 - P_in

2. Total Data Volume for 256×256 Image: N = 256 x 256 = 65,536

Output

| S.No | Output Parameter | Expected / Calculated Output | Simulated / Measured Output | Result / Deviation |
| --- | --- | --- | --- | --- |
| 1 | Processed Frame Size | 65,536 Bytes | 65,536 Bytes | 0% Byte Loss |
| 2 | Sample Inversion (Input: 0) | P_out = 255 | P_out = 255 | 0% Error |
| 3 | Sample Inversion (Input: 100) | P_out = 155 | P_out = 155 | 0% Error |
| 4 | Sample Inversion (Input: 255) | P_out = 0 | P_out = 0 | 0% Error |
| 5 | Image Inversion Accuracy | 100% | 100% | 0% Deviation |

Figure 1: Example of image intensity inversion

Result

The image intensity inversion hardware circuit was successfully designed, modeled, and simulated with Verilog RTL in Xilinx Vivado. Simulation results in Vivado Simulator (XSIM) confirm that all 65,536 pixels of a 256×256 grayscale image frame were accurately transformed according to the inversion equation (Pout = 255 - Pin), mapping low intensities to high intensities with zero arithmetic error and validating complete functional correctness.