Experiment 9: Design of Fully Connected Neural Network using RTL

Objectives

  - To create and simulate a Fully Connected (FC) neural network layer with five input neurons (x0–x4), five weight inputs (w0–w4), and one output neuron (y) with RTL (Verilog HDL).

  - To realize fixed-point signed 8-bit x 8-bit parallel multipliers, a 5-input adder tree accumulator (sum), and a 16-bit signed bias addition stage.

  - To construct a comprehensive Verilog testbench (tb_fully_connected_5in) applying positive, scaled, and signed 2's complement test vectors.

  - To carry out simulation of in Vivado Simulator (XSIM), record output waveforms, and verify 100% computational accuracy against manual theoretical MAC calculations.

Software Required

Xilinx Vivado Design Suite, Vivado Simulator (XSIM), Verilog HDL, Windows / Linux Operating System.

# Procedure

Step 1: Project Creation — Open Xilinx Vivado and create a new RTL project. Select the target FPGA device or board.

Step 2: Module Architecture & Ports Declaration — Create a Verilog module named fully_connected_5in for the Fully Connected layer. Declare five 8-bit signed input ports (x0 to x4), five 8-bit signed weight ports (w0 to w4), a 16-bit signed bias input (bias), and an 18-bit signed output (y).

Step 3: MAC Logic Implementation — Implement parallel 8-bit x 8-bit signed multipliers (m0 = x0 * w0 through m4 = x4 * w4), an accumulator tree (sum = m0 + m1 + m2 + m3 + m4), and a bias adder (y = sum + bias).

Step 4: Testbench Development — Create a Verilog testbench file (tb_fully_connected_5in) instantiating fully_connected_5in. Apply test cases for: (1) All-ones weights with bias 10, (2) Scaled weights x2 with bias 0, and (3) Mixed signed negative inputs with bias 5.

Step 5: Simulation & Result Verification — Launch Vivado Simulator (XSIM) and execute behavioral simulation. Inspect output waveforms for each test case, record y values, and compare them against manual theoretical calculations.

# Program

## 1. Verilog RTL Code: Fully Connected Layer

module fully_connected_5in (
    input  signed [7:0] x0, x1, x2, x3, x4,   // 5 inputs
    input  signed [7:0] w0, w1, w2, w3, w4,   // 5 weights
    input  signed [15:0] bias,                // single bias
    output signed [17:0] y                    // output
);
    wire signed [15:0] m0, m1, m2, m3, m4;
    wire signed [17:0] sum;

    assign m0 = x0 * w0;
    assign m1 = x1 * w1;
    assign m2 = x2 * w2;
    assign m3 = x3 * w3;
    assign m4 = x4 * w4;

    assign sum = m0 + m1 + m2 + m3 + m4;
    assign y   = sum + bias;

endmodule

## 2. Verilog Testbench Code

`timescale 1ns / 1ps
module tb_fully_connected_5in;
    reg  signed [7:0] x0, x1, x2, x3, x4;
    reg  signed [7:0] w0, w1, w2, w3, w4;
    reg  signed [15:0] bias;
    wire signed [17:0] y;

    // Instantiate the DUT
    fully_connected_5in uut (
        .x0(x0), .x1(x1), .x2(x2), .x3(x3), .x4(x4),
        .w0(w0), .w1(w1), .w2(w2), .w3(w3), .w4(w4),
        .bias(bias),
        .y(y)
    );

    initial begin
        // Test Case 1: Positive inputs, all-ones weights, bias = 10
        x0 = 8'd10;  x1 = 8'd20;  x2 = 8'd30;  x3 = 8'd40;  x4 = 8'd50;
        w0 = 8'd1;   w1 = 8'd1;   w2 = 8'd1;   w3 = 8'd1;   w4 = 8'd1;
        bias = 16'd10;
        #10;

        // Test Case 2: Scaled weights x2, bias = 0
        x0 = 8'd5;   x1 = 8'd10;  x2 = 8'd15;  x3 = 8'd20;  x4 = 8'd25;
        w0 = 8'd2;   w1 = 8'd2;   w2 = 8'd2;   w3 = 8'd2;   w4 = 8'd2;
        bias = 16'd0;
        #10;

        // Test Case 3: Mixed signed negative values, bias = 5
        x0 = -8'd10; x1 = 8'd20;  x2 = -8'd5;  x3 = 8'd15;  x4 = 8'd10;
        w0 = 8'd1;   w1 = 8'd1;   w2 = 8'd1;   w3 = 8'd1;   w4 = 8'd1;
        bias = 16'd5;
        #10;
        $stop;
    end
endmodule

# Formulas

1. Fully Connected Neuron MAC Formula: y = (sum_{i=0}^{4} x_i * w_i) + bias

2. Individual Test Case Arithmetic Verification:

• Test Case 1 (Positive inputs, all-ones weights): sum = (10x1) + (20x1) + (30x1) + (40x1) + (50x1) = 150; y = 150 + 10 = 160

• Test Case 2 (Scaled weights x2): sum = (5x2) + (10x2) + (15x2) + (20x2) + (25x2) = 150; y = 150 + 0 = 150

• Test Case 3 (Mixed signed negative inputs): sum = (-10x1) + (20x1) + (-5x1) + (15x1) + (10x1) = 30; y = 30 + 5 = 35

3. Bit-Width Precision & Guard Bit Analysis: 8-bit x 8-bit signed product yields 16-bit signed terms (m0..m4). Accumulating 5 16-bit terms requires ceil(log2(5)) = 3 guard bits, giving an 18-bit accumulator (sum). Adding a 16-bit bias yields an 18-bit output (y) preventing overflow.

Output

| S.No | Test Scenario / Parameter | Expected Theoretical Output | Simulated Hardware Output | % Deviation / Status |
| --- | --- | --- | --- | --- |
| 1 | Test Case 1 (Positive Inputs) | y = 150 + 10 = 160 | y = 160 | 0% Error |
| 2 | Test Case 2 (Scaled Weights x2) | y = 150 + 0 = 150 | y = 150 | 0% Error |
| 3 | Test Case 3 (Mixed Negative Values) | y = 30 + 5 = 35 | y = 35 | 0% Error |
| 4 | Accumulator Precision | 18-bit Signed Accumulator | No Arithmetic Overflow | Verified |

Figure 1: sample fully connected network output

## Result

The Fully Connected (FC) Neural Network layer with five input neurons was successfully designed, implemented, and simulated with RTL Verilog in Xilinx Vivado. The circuit executed 5 parallel signed 8-bit multiplications, accumulated product terms into an 18-bit register, and added the 16-bit bias value. Simulation waveforms across all 3 test cases (Positive values, Scaled weights, and Mixed signed negative inputs) matched theoretical MAC calculations with 0% error, confirming 100% functional accuracy of the hardware neuron for AI acceleration.